Reagent:

- M280 beads

- 2 x B&W buffer: 1 mM EDTA, 2 M NaCl, 10 mM Tris-HCl pH7.5, 0.01% Tween20 (工作浓度1x)

- Low salt buffer: 0.1M NaCl

- 10 x HSWB buffer: (工作浓度1x)

|  | 15 ml | 50 ml | Final con. |
| --- | --- | --- | --- |
| 1 M Tris-7.5 | 1.5 ml | 5 ml | 100 mM |
| 0.5 M EDTA | 300 ul | 1 ml | 10 mM |
| 5 M NaCl | 3 ml | 10 ml | 1 M |
| 10% Tween-20 | 150 ul | 0.5 ml | 0.1% |
| H2O | Up to 15 ml | Up to 50 ml |  |

- Solution A

|  | 1 ml | Final |
| --- | --- | --- |
| 1 M NaOH | 100 ul | 0.1 M |
| 5 M NaCl | 10ul | 50 mM |
| H2O | 890 ul |  |

- Solution B

|  | 1 ml | Final |
| --- | --- | --- |
| 5 M NaCl | 20 ul | 0.1 M |
| H2O | 980 ul |  |

- 50 mg/ml yeast tRNA.

- 10x Lysis buffer (LB buffer): 我用来重悬和冻存细胞的buffer：200mM Tris-HCl，1M NaCl, 50 mM MgCl2, (0.1% BSA (10% BSA 1:100加), RNase inhibitor 1:1000, 现加)。可以换成你们的buffer

- RNA S2: EU labeled果蝇RNA，如果biotin pull-down量太少了，可以在孵育beads和样品时适量补一些这个RNA，促进pull-down

Oligos:

这是我用来做bulk的oligo，可以换成你们的建库方式

- TSO-PCR: AAGCAGTGGTATCAACGCAGAGA

- TSO-﻿CATGG: AAGCAGTGGTATCAACGCAGAGACATGG

- TSO-N9: AAGCAGTGGTATCAACGCAGAGACATGNNNNNNNNN

- TSO-oligo: dAdAdGdCdAdGdTdGdGdTdAdTdCdAdAdCdGdCdAdGdAdGdAdCdAdTdGrGrGrG (仅最末端三个G为rG)

- TSO-LNA:

dAdAdGdCdAdGdTdGdGdTdAdTdCdAdAdCdGdCdAdGdAdGdAdCdAdTdGrGrG+G（注：TSO-LNA最末端是LNA-modified guanosine (+G) ）

After droplet demulsion

+ 40 ul PK buffer to the cells (35 ul PK + 5 ul 20mg/ml PK). 37 C 30 min (能解交联即可).

- + 1 ul Exo1 to the sample, 5ul 10x Exo1 buffer and 4 ul H2O. 37C 30 min (remove excess random primers).

- 1.8 x XP beads purification, elute in 10 ul (这个体积根据下一个反应可调节).

TSO 2nd RT for RNA-DNA hybrid

- Prepare RT mix (total 20 ul, hybrid 10 ul), Pre-warm the 1 x B&W buffer in 50 C.

| 5x FS | 4 |
| --- | --- |
| 10 mM dNTP | 1 |
| 10 uM TSO-oligo or TSO-LNA | 1 |
| 0.1 M DTT | 1 |
| Rnase in | 0.5 |
| SS iv | 0.5 |
| H2O | 2 |

- 23C 5’, 42C 5’, 50C 5’ (Note: the best temperature for ss iv is 50C, adjust to your RT reactions).

- Add H2O up to 50 ul for pull down. (pull down in 200 uL PCR tube, 50 ul total)

Puriﬁcation of 5EU Labeled RNA/cDNA

- 10 ul M280 are firstly washed with 400 ul 1 x B&W twice (quick wash).

- Then wash beads with 200 ul washing solution A twice and solution B once (quick wash).

- Resuspend the beads in 500 ul 1 x HSWB supplemented with 10 ul 50 mg/ml yeast tRNA. room temperature for 30 min with end-over-end rotation.

- Beads were then washed three times with 500 uL 1x HSWB per reaction. Resuspend the beads in 50 ul 2x WB.

- Transfer 50 ul block beads into the TSO-RT sample (now 1x B&W). rotate room temperature for 30 min.

- Beads were washed once with 180 uL 50 C 1X B&W for 5 min (PCR仪). washed three times with 180 uL B&W at room temperature, and once with 200 uL low salt buffer. (先把PCR管放到磁力架上，等待beads吸附到磁力架一侧，吸去上清。再加wash buffer，PCR管从磁力架上取下来，旋转PCR管让beads从靠近磁力架一侧转到靠近自己这一侧，再放到磁力架上，beads此时从原来的那侧通过磁力架吸附到另一侧，达到wash的效果。50C 5min的wash可以先这样操作然后PCR仪保温5min。)

- Resuspend the beads in 5 ul H2O. Take half. (这个体积根据下一个反应可调节)

Second strand synthesis

- NaOH lysis: + 3 ul H2O, 1 ul 1 M NaOH, 70C 5 min. then + 1 ul 1 M HCl.  (水解去除RNA。这里我利用了scNT-seq的方法（Qi Qiu et al. 2020 Nat Method），可以促进template switch失败的cDNA的捕获。也可以先按你们的来)

| 10x NEB 2 | 4 |
| --- | --- |
| 10 mM dNTP | 2 |
| Klenow Fragment | 1.5 |
| 100uM TSO N9 | 2 |
| 100uM TSO-CATGG | 1 |
| cDNA | 10 |
| H2O | 19.5 |

25C 10’, 37C 20’, 50C 10’.

No Fragmentation here. (片段本身偏短，不需要fragmentation)

Purify with 1 x DNA beads, elute in 20 ul, take half do PCR. (我一般会留一半样，因为可能需要两步PCR)

Final PCR

- PCR mix

|  | μl |  |
| --- | --- | --- |
| 2x PCR mix | 12.5 | 53 |
| 10 μM D701-TSO | 1.125 | 4.2 |
| 10 μM D5 | 1.125 |  |
| DNA | 10 |  |
| Total | 25 |  |

PCR for 20 cycles.

- Optional:

  - 考虑到insert可能偏短，所以我有时会根据第一次PCR胶图判断是否需要做两步PCR：第一轮用到TSO-PCR引物

|  | μl |  |
| --- | --- | --- |
| 2x PCR mix | 12.5 | 53 |
| 10 μM TSO-PCR | 1.125 | 4.2 |
| 10 μM D5 | 1.125 |  |
| DNA | 10 |  |
| Total | 25 |  |

  - PCR for 10 cycles, then PCR using indexed primers D5-D7 for 10 cycles. Run agarose gels to check library size.

这是我当时的library: